For Academic Use Only                                                                           
                                                                                                                                                  [image: image10.png]


              

Lab 2: Architecture Wizard and PACE Lab
Targeting XUP Spartan-3E
Architecture Wizard and PACE Lab

Introduction 

This instructor-led demonstration introduces the Architecture Wizard and PACE.

Objectives 

After completing this lab, you will be able to:

· Use the Architecture Wizard to configure a DCM component

· Instantiate the DCM component into the design

· Use PACE to assign pin locations

· Implement the design and confirm that the pin assignments were used

· Download and test design in hardware

Procedure 

This demonstration comprises four primary steps: you will use the Architecture Wizard to configure a DCM, instantiate the DCM into a VHDL/Verilog design, use PACE to assign pin locations, implement and download the design, and then finally test it in hardware. Below each general instruction for a given procedure, you will find accompanying step-by-step directions and illustrated figures that provide more detail for performing the general instruction. If you feel confident about a specific instruction, feel free to skip the step-by-step directions and move on to the next general instruction in the procedure.

Note: If you wish to review this software demonstration at a later time, you can download the files from the Xilinx University Program site at http://www.xilinx.com/univ
Design Overview

This lab will make use of the UART Real-Time clock design and detailed information can be found in the UART_real_time_clock.pdf provided with the PicoBlaze distribution.  This section will attempt to organize and highlight the feature of the design.

 The design implements a real-time clock maintaining time in hours, minutes and seconds together with the ability to set an alarm.  The unusual feature of the design is that a UART serial communication is used to set and observe the time/alarm sending simple text commands and messages via a utility such as hyperterminal.

The design understands some simple ASCII commands and even supports some editing during their entry using the backspace key on your keyboard.  A command is only completed when a carriage return is entered.  The design is ready to accept a command when the “KCPSM3>” prompt is displayed.  

The “uclock” program provided with the distribution is able to interpret upper and lower case characters by converting commands (see documentation for details) to upper case before analyzing them.  Incorrect commands will result in a “syntax error” message and incorrect time values will be indicated by an “Invalid Time” message.  Although it is unlikely to occur when using hyperterminal, an “overflow error” message will be generated if commands are transmitted faster than the design can process them (ie. UART receiver buffer becomes full).  

The design requires a 55 MHz clock.  Since the Spartan-3E board includes a 50 MHz oscillator, you will use the Architecture Wizard to generate a DCM with a 55 MHz output and instantiate it into the design.

Use the Architecture Wizard to Configure a DCM
Step 1

[image: image10.png]Open an existing project. 
If you have closed the ISE™ Project Navigator, select Start ( Programs ( Xilinx ISE 8.2i ( Project Navigator

· Select File ( Open Project in the Project Navigator

· Browse to one of the following directories and select arwz_pace.ise
· Verilog users: c:\xup\fpgaflow\labs\veriloglab2\arwz_pace 

· VHDL users: c:\xup\fpgaflow\labs\vhdl\lab2\arwz_pace

· Click Open
[image: image11.png]


This version of the design is missing a DCM component. Use the Architecture Wizard to configure a DCM component to output a clock at 55 MHz. 
· In the Processes for Source window, double-click Create New Source
If you do not see the Create New Source process, ensure that an HDL source file is selected in the Sources in Project window.

· In the New Source window, select IP (CoreGen & Architecture Wizard) and enter my_dcm as the file name

· Click Next

· In the Select Core Type window, expand FPGA Features and Design (Clocking ( Spartan-3E, Spartan-3A and select Single DCM SP v8.2i (Figure 2-2)


[image: image1]
Figure 2-2. Architecture Wizard Selection Box

· Click Next, and click Finish

· In the Xilinx Clocking Wizard – General Setup window, set the following options (as shown in Figure 2-3):

· CLK0, CLKFX and LOCKED boxes: Checked

· RST box: Unchecked

· Input Clock Frequency: 50 MHz
[image: image12.png]



Figure 2-3. Xilinx Clocking Wizard – General Setup Window

Note: Specifying a frequency in the Clocking Wizard is done so that the wizard knows whether the DCM is in High or Low frequency mode. Based on the input frequency, the wizard also calculates output frequencies to ensure these frequencies are within the specified range. In addition, if the CLKFX output is used, the input frequency is used to calculate the output frequency at the CLKFX and to calculate estimated jitter.

· Click <Next>

In the Xilinx Clocking Wizard – Clock Buffers window (Figure 2-4), keep the defaults and click <Next>
[image: image2.png]ilinx Clocking Wizard - Clock Buers
I~ Closk Buffer Setfings

= e il B ol et ek s
" Customize bufers

Input [0 Input 1 Visw/Edit Buffer
cua GlbalBuifer
k. (GlsbalBlifer

More Info

Bk Cance





Figure 2-4. Xilinx Clocking Wizard – Clock Buffers Window

· In the Xilinx Clocking Wizard – Clocking Frequency Synthesizer dialog, enter 55 Mhz as the output frequency, click <Next> and then <Finish>.

Note: If you do not see my_dcm.xaw in the design hierarchy, manually add by going to Project ( Add Source.


[image: image3]
Figure 2-5. Specifying the DCM output frequency

Notice that a new file (my_dcm.xaw ) is added as a Source in the project (figure 2-6). This source file will not be included in the design hierarchy until the component has been instantiated into one of the HDL source files.


[image: image4]
Figure 2-6. DCM listed in design hierarchy

Instantiating the DCM into a Verilog Design
Step 2a

VHDL users can skip to the next section…Step 2b

[image: image13.png]


Now that you have created the necessary files, you can instantiate the DCM component into your design. You will copy-and-paste the text from the Instantiation Template into uart_clock.v and connect the signals.
· With the my_dcm.xaw file selected, go to the Processes for Source window and double-click View HDL Source to examine the source code generated by Architecture Wizard

[image: image14.png]


If the file does not appear in the text editor, double-click View HDL Source again.

This file contains the following instantiated components: an IBUFG, a DCM, and two BUFGs.

The input clock CLKIN_IN drives the IBUFG, which is connected to the DCM. The two output clocks are driven onto BUFG components.

All of the DCM attributes are passed through Verilog meta-comments.

· In the Sources in Project window, double-click uart_clock.v to open the source code in the text editor

· Select my_dcm.xaw in the Sources in Project window

· In the Processes for Source window, double-click View HDL Instantiation Template to open the instantiation template in the text editor

[image: image15.png]


If the template does not appear in the text editor, double-click View HDL Instantiation Template again.

· In the Instantiation Template my_dcm.tfi, copy the instantiation module and paste into uart_clock.v under the comment // Insert DCM component declaration here
· Complete the instantiation by filling in the port connections as follows:

my_dcm inst_my_dcm (

    .CLKIN_IN(clk), 

    .CLKFX_OUT(clk55MHz), 

    .CLKIN_IBUFG_OUT(), 

    .CLK0_OUT(), 

    .LOCKED_OUT(lock)

    );

Note: The clkin_ibufg_out port is an output that is present to support designs that use the RocketIO™ transceivers.  Because you are not targeting Virtex-II Pro™ devices, this port will be connected to a dummy signal.

· Add a signal declaration for the 55 MHz output of the DCM under the comment 

//  Signals for DCM, as follows:

wire clk55MHz;


note: the uart_clock.v design has been updated so that all of the clock signals (clk) 

have been renamed to reflect the new clock (clk55MHz).

· Add an output pin for lock in the top-level module as follows:

module uart_clock

(
tx,


rx,


alarm,


clk,


lock);

output
tx;

input 
rx;

output 
alarm;

input 
clk;

output   lock;



note: this lock output pin will drive the led1 on the Spartan-3 board, and is driven by 

the lock signal on the DCM.  This will indicate to the user that the DCM has successfully 

locked onto the 50 MHz clock signal from the on-board oscillator.

· Click File ( Save to save the file

Notice that the source file my_dcm.xaw is now inserted into the correct place in the design hierarchy.

Instantiating the DCM into a VHDL Design
Step 2b

[image: image16.png]


Now that you have created the necessary files, you can instantiate the DCM component into your design. Copy-and-paste the text from the Instantiation Template into uart_clock.vhd and connect the signals.

· With the my_dcm.xaw file selected, go to the Processes for Source window and double-click View HDL Source to examine the source code generated by Architecture Wizard

[image: image17.png]


If the file does not appear in the text editor, double-click View HDL Source again.

This file contains the following instantiated components: an IBUFG, a DCM, and two BUFGs.

The input clock CLKIN_IN drives the IBUFG, which is connected to the DCM. The two output clocks are driven onto BUFG components.

All of the DCM attributes are passed through VHDL generics. 

· In the Sources in Project window, double-click uart_clock.vhd to open the source code in the text editor

· Select my_dcm.xaw in the Sources in Project window

· In the Processes for Source window, double-click View HDL Instantiation Template to open the instantiation template in the text editor

[image: image18.png]


If the template does not appear in the text editor, double-click View HDL Instantiation Template again.

· In the Instantiation Template my_dcm.vhi, copy the component declaration (begin at COMPONENT my_dcm and end after END COMPONENT;) and paste into uart_clock.vhd under the comment -- Insert DCM component declaration here
· In the HDL Instantiation Templatemy_dcm.vhi, copy the component instantiation (begin at 
Inst_my_dcm: my_dcm until the end of the file) and paste into uart_clock.vhd under the comment -- Insert DCM component instantiation here
· Complete the instantiation by filling in the port connections as follows:


    
Inst_my_dcm: my_dcm PORT MAP(



CLKIN_IN                   => clk,

                      CLKFX_OUT              => clk55MHz,



CLKIN_IBUFG_OUT => open,



CLK0_OUT                 => open,

LOCKED_OUT           => lock

);

Note: The clkin_ibufg_out port is an output that is present to support designs that use the RocketIO™ transceivers.  Because you are not targeting Virtex-II Pro™ devices, this port will be connected to a dummy signal.

· Add a signal declaration for the 55 MHz output of the DCM under the comment --  Signals for DCM, as follows:

signal clk55MHz : std_logic;

note: the uart_clock.vhd design has been updated so that all of the clock signals (clk) 

have been renamed to reflect the new clock (clk55MHz).

· Add an output pin for lock in the entity as follows:

entity uart_clock is

    Port (    

              tx : out std_logic;

              rx : in std_logic;

             alarm : out std_logic;

             clk : in std_logic;

            lock : out std_logic

           );

end uart_clock;



note: this lock output pin will drive the led0 on the Spartan-3 board, and is driven by 

the lock signal on the DCM.  This will indicate to the user that the DCM has successfully 

locked onto the 50 MHz clock signal from the on-board oscillator.

· Click File ( Save to save the file

Notice that the source file my_dcm.xaw is now inserted into the correct place in the design hierarchy.

Use PACE to Assign Pin Locations
Step 3

[image: image19.png]


You have completed the design by adding a DCM.  

Most FPGA designs have pinout requirements before the design is complete.  PACE makes it easy to assign pins and check that the chosen pinout conforms to all DRC rules regarding I/O standards and I/O banks.

In this lab, you will use PACE to assign locations to the pins in the design. 

· In the Sources in Project window, select the top-level design file uart_clock.vhd/.v
· In the Processes window, expand User Constraints and double-click Assign Package Pins to open PACE  

Click “yes” when asked to add a UCF file to the project.  The design must be synthesized before PACE can be launched.

· Browse through the Design Object List window, and review the listed signals.  Note that these are the inputs and outputs of the design.

· In the Loc column next to each signal, enter the pin numbers to interface the inputs/outputs of the design to the Digilent Spartan-3E board as follows (Figure 2-7) and then save the design:

· clk     : connected to 50 MHz oscillator
· lock   : connected to led0 

· alarm : connected to led1

· rx      : connected to pin that receives serial data from Maxim MAX3232

· tx      : connected to pin that transmits serial data to Maxim MAX3232

Note: Refer to the Digilent Spartan-3E Users’ Manual for the pinouts

· In the dialog, select XST Default: <> as the I/O Bus Delimiter and click OK
[image: image20.png]Presentation1]
@) Fle Edt Vew Insert Format Tools Side Show Window Help 8%

Ded &V [ Xilinx Clocking Wizard - General Setup

New Side... Side Layouk... Apply Desian oI —
Times New Roman BT — CLKFB CLK0 —
CLK180-— I~ B
CLK270+— I~
CLKDV— I~
CLK2X— I~
CLK2X180 — [~
CLKFX+— ¥
CLKFX180 — [~
I~ —RST
STATUS — I~
LOCKED —— ¥
T ——
Input lock Fiequency Phase Shit
BT G Cone || Twe NONE ]
vae [0 oo 0000Dsgess
CLKIN Souce Feedback Souce
& Evtenal © Intenal CBdemad  © lemal O None
& Single 3
© Differenta o
Feedback Value
[EEmmE] O
¥ Use Duty Cycle Corrction
Advanced. More Info
Cancel 3
| I

agosapes- \ N IO 4| 2~ L-A-=
Side 101 Defout Desian





View your pin assignments in relation to the internal logic. 

· In the Device Architecture window, zoom in until you can see individual pin numbers (Figure 2-8)
The colored bar alongside the I/O pins indicates which pins are in the same I/O bank. You can easily see which pins have been assigned to the same bank.


[image: image5]
Figure 2-8. Device Architecture Window

· Click each colored I/O pin. The corresponding pin will be selected in the Design Object List window

· In the Design Object List window, confirm that the pin assignments have been changed

· Click File ( Save to save these pin placements

· Click File ( Exit to close PACE

· In the Processes pane of the Project Navigator, expand User Constraints and double-click Edit Constraints (Text) to view the constraints created in the uart_clock.ucf file through PACE. View the text version of the UCF file to confirm that the constraints were written to the file

Verify Pad Report and start Hyperterminal Session
     Step 4

[image: image21.png]E Ele £t tiew Project Soure

Process window_tielp

[FL]
LLRHEABAimE D LN VIHTE BAAANLIQO

s % [ FPGA Design Sunmay ]

Sources for [ Syribesimplmentation v

Z5Design Ovenien

Eawapace

& Ccassteiigi2)

Fuart_clock (uatt_clockv)

processor - kepsm (kopsm.v)
program_am - uclock (LUCLOCK.Y)

2 instance_name - my_dom (my_dem xam)
st - uatt_t (uar_tev)

[

eceive - uattx uar_1xv)
F)uart_clock.ucf (uat_clock.uch)

Sunmary
108 Propertes
Tining Constiaints
Picut Repart
Clock Repart
GEnors and Warings
Sprhesis Messages
Trastaion Messages
Map Messages
Place and Routs Messages
Tining Messages

e |

Pracesses

Bigen Messages
1 Curert Messages
x | vetsidRepors

Pracesses:

Synthesis Repart

<l

‘Add Esstng Source
Create New Source,
View Design Sunmay
Design Uties
User Constaints

&

@

@

E|

Create Tining Constaints
‘Assign Package Pins
Create Area Constiants
Edit Constraints (Text)

22,4\ Synthesize -XST
£ 2@Implement Design
& 2@ Generate Programming File

[ Progrsmming Fie Generaton Repot

@ Genersts PROM.ACE, orJTAG Fie

@ Configure Device (MPACT)
UndateBisteam it Processor Dats

@ Analyze Design Using Chipscope:

Translaton Regot
Map Repart

Place and Route Regot
Staic Tining Repart
Bigen Repart

&3 Secondany Repars
DXploer Regort

T v

Number of Sice Fiip Flops 147 9312 1%

| Nober o i LUTs

I New Source Wizard - Select IP

(£ Automoive & Industial
8 (ZIBasic Elements
8 (23 Communication & Networking
(23 Digal Signal Processing
= E3FPGA Features and Design

= &3 Clocking

= 3 Spartan 3, Spartan-34.

¢ Board Deskew with an Ineinal Deskew (DCM SP) 8.2
Cascading in Seriss with Two DCM 5P V8.2
Clock Forwarding / Board Deskew (DCM SF) v8.2i

N

Project Propertes
Enable Enhanced Design Summary
O Enable Message Fitering

00 Display Incremental Messsages
Enhanced Design Sunmery Corterts

O Show Partiion Data

O Show Enors

O Show Warings

00 Show Faiing Constaints

O Show Clack Repart

x

< Clock Switching with Tw DCM SPs v8.21 E3
d E3
6 (IMath Functions 5

6 (ZIMemories & Storage Elements
6 (23 Standard Bus Interfaces

‘Surihesis Repat Curert Mon Sep 18155825 2006 327Wavmn s

| Tartaon Repor | Cunent | ion Sep 18 155832 2008 \ o \
| iapBepar | Cunent [MonSep 181558382008 |0 \ Py | lfos
2 | Place and Route Report | Curtent [MonSep 181556582006 |0 I | Zios ~
(B | Y r—

<

Process "Generate Post-Place § Route Static Timing” completed successfully

Started :

"Generate Programming File".

INFO: PhysDesignRules: 772 - To achieve optimal frequency synthesis performance
with the CLKFX and CLEFX180 outputs of the DCH comp
instance_name/instance_name/DCH_INST/ instance_name/DCH_INST, consult the

device Interactive Data Sheet.

Process "Generate Programming File" completed successfully

<1 |
[£] Consoe | @Evors | g\Warings | @ TelConsole | igg Fndin e |




Implement the design and review the PAD report to confirm that the pin assignments were obeyed.  Start a hyperterminal session. 
· Make sure that the top-level design file uart_clock.vhd/.v is selected in the Sources in Project window

· In the Processes for Source window, expand the Implement Design process and expand Place & Route
· Double-click Pad Report 

The Project Navigator automatically determines which processes must be run, and it will open the report once the Place & Route process is completed.

· Scroll down in the report and confirm that the pin numbers for the I/O signals match the assignments you made

· Open a hyperterminal session by going to Start ( All Programs (Accessories ( Communications ( HyperTerminal

· Give the session a name, click <OK>, and specify COM1 as the port connection.

· Click the Configure button and specify the following parameters for the port settings.  Click   <OK> when finished.

· Baud rate of 38400

· 8 data bits

· No parity bits

· 1 stop bit

· No flow control

[image: image22.png]Show Window Help Type aquestion for help '+ | X

Apply slide layout:

ides

Text Layouts

E# Xilinx Clocking Wizard - Clock Frequency Synthesizer,

 Valid Ranges for Speed Grade -4
Dfs Mode Fin (MHz) Fout (MHz)

Low 0200- 332000 5000- 326,000

High 0200-333000 5000- 333000

InputsforJiter Calculstons
Input Clock Frequency: 50,000 MHz

® Use oulput hequency Content Layouts

[ ——CT P

© Use Multly (M) and Divide (D) values

2 00|

=
i

 Generated Dutput =

M Output Period Jitter (unit | Period Jitter
Freq (MHz) interval) (pk-to-pk ns)

11 % 005 0%

Text and Content Layouts

Other Layouts

Click to add notes o

Shaw uhen inserting new sides

Slide 1 of | Default Design Engish (U1.5.) 223





Figure 2-9. Settings for Serial Port Communications

· Click the settings tab and then ASCII Setup tab and then click so that a check mark appears next to the Append line feeds to incoming line ends option, and then click <OK>.  Click <OK> again to exit the properties dialogue.

[image: image23.png][t st yon fmert Fomat Lok Table widow telo

D@ ERY I Ead oo 0@ HEREH BT o - 0

2
[~ e =
e Edh Vew Cal TrnserHeb
- _low e on®
c0M1 Properties
[Fotseting: |
: E—
E act
S
Pl conko
Disconnected [auodetect | Autodetect | SCROLL | CAPS | um | Copiure | Frinteche
HEER
[oon- Iy & | asostapes- N NCOJOE 4l &-L-A ELY ]

«loln|«

[Page 18 Seci _ iolis_[mis' ns Gold _|rec [ b [ [ | O |





Figure 2-10. ASCII Settings for Serial Port Connection

Download and the Design
Step 5

[image: image24.png][t st yon fmert Fomat Lok Table widow telo

lDsRERY | et o-- e BEE

= S

E\@ o« 100% v\@

N s errerermmes
E__los ~5 om &

HRRKK

" Comnect To| Setings |

[2][x]

N ® Teminalkeys O

Backspace key sends

“ @okH ODel O

Emuaton:

N Auto detect
L Tebetteminal ID:[ANST

Backscrol bufer fnes: (500

[ Play sound when connecti

- Input Transatio,

ASCII Setup

Functon, irow, and ctl keys act as

STl Sending
[ Send ine ends with ine feeds
[ Echo typed characters locally

e cl: [0 nlsoons.
st doly [0 misoncs

ASCIl Reseiving

ppend ine feeds o incoming i ends
[ Force incoming data to 7-tit ASCII

3 lnes that exczed teminal widh

=[o[a=

Comected 004:44  Autodetect  Autodetect O A oM e P

o= s &

agoshopes - N N OO E 4| &-L-A-=

ELL

«loln|«

[Page 18 Seci o4 [ms7 ol Golt _|rec [ b e [ | G





Generate the bitstream and download to the FPGA

· Highlight uart_clock.vhd and double-click on Generate Programming File to generate the bitstream that will get downloaded to the FPGA.

· When the process is complete, expand Generate Programming File and double-click on Configure Device (iMPACT).

· With Configure Devices using Boundary-Scan (JTAG) selected, click <finish>.

Note: Click <OK> when the Boundary-Scan Chain Contents Summary dialog appears.

· When the Assign New Configuration File dialog box appears, select uart_clock.bit file for the xc3s500e (first device in JTAG chain) and click <open>.  

Note: A warning message will appear indicating that the startup clock has been changed to the JTAG clock.  Click <OK>.

· Click Bypass for the other devices.


[image: image6]
Figure 2-11. JTAG Chain with assigned configuration files

· Connect the JTAG download and serial communications cable to the Spartan-3E board and power it up.

· Right-click on the xc3s500e in the iMPACT window, select Program, and click <OK> in the Programming Options dialog.  

Note: You should now see the KCPSM3> prompt in the hyperterminal window.


[image: image7]


Figure 2-12. Serial Communication with PicoBlaze

Operating the UART Real-Time Clock
    Step 6

[image: image25.png]UP/Markets/PLDs/Workshops/courses/v8; low/xupsp3e/labsolutions/verilog/lab2/arwz_pace.ipf - [Boundary Scan]

=&

Fle Edi View Operations Options Qutput Debug Window Help

BE %BBX & i (#BO W W

“23Bounday Sean
23SloveSerial
23selectMaP
“23Desktop Configuaton
[S)systemace o

MPACT Modes

[E3

o F S P -

xc3ss00e xefoas xc2c8da
uart_clock bt et el

0.

\Avalble Operatons are:
= Progiam

=iy

=Gt Device ID

1= Get Device Signature/Usercods
‘= Check Idcods

= Read Stalus egister

IMPACT Pracess Operations & Sy Son

INFO:1MPACT:2215 - Status register values:

>

INFO:iMPACT - 0011 0111 1011 1000 0000 000D 0000 0000
INFO: iMPACT:579 - '1': Completed downloading bit file to device.
INFO: iMPACT:580 - '1':Checking done pin ....done.

'1': Programmed successfully.
PROGRESS_END - End Operation.

Elapsed time = 1 sec.
& I 3
OQuiput | Enor | Waming |

Configuration | Platform Cable USB | 6 Mz | usbhs

Ready





You will issue commands to operate the UART real-time clock, as specified in the UART_real_time_clock.pdf file.

· Enter the command “time” at the command prompt to display the current time in the form of hh:mm:ss


[image: image8]
Figure 2-13. Display of Current Time

· Enter the command “alarm” at the command prompt to display the current alarm time in the form of hh:mm:ss

Note: the alarm is inactive


[image: image9]
Figure 2-14. Display of alarm time and status

· Enter the command “alarm on” for the alarm to become active

· Enter the command “alarm 00:00:30” to set the alarm to 30 seconds

· Enter the command “time 00:00:00” to set the time

Note: You should notice that led1 on the Digilent Spartan-3E board will light up once the alarm goes off.

· Enter the command “alarm off” to shut off the alarm.

Note: You should notice that the led0 shuts off, indicating that the alarm has been disarmed.

Conclusion 

In this demonstration, you used the Architecture Wizard to configure a DCM component and you instantiated the component into the design. You used PACE to make pin assignments. Finally, you implemented the design and tested in hardware.

� EMBED Word.Picture.8  ���











� EMBED Word.Picture.8  ���








!





!











� EMBED Word.Picture.8  ���

















� EMBED Word.Picture.8  ���














!





!


















































!





!











� EMBED Word.Picture.8  ���











!





!














� EMBED Word.Picture.8  ���








� EMBED Word.Picture.8  ���











This material exempt per Department of Commerce license exception TSU
[image: image26.png]Bl Edt Vew [0Bs Areas Took Window Help

DEE (& o s L0|Hs yE8s 240 Es 0 [ XQaap 00NN EEEECEE
) De owse [m] i Device ecture fo 00e-4-fg320
3 10 Pins.
Glabal Logic
L]
aw >
5] 0 0P
170 Nome] 170 Direstio]_Loc_[Bark] 170 5td
G Oupi e BARIITL
S © ANk LVOMDSE
ok Oupi 12 BANKWVITL
A ot 7 BAKwWITL
" Oupdmid BANKLVTTC
< m Package View ), Architecture View / Kl




Architecture Wizard & PACE
www.xilinx.com/univ
2-8
Lab
xup@xilinx.com
 


[image: image27.png]AXUP\Markets\PLDsWorkshopsicoursesivB1_fpga_flowbxupsp3eMabsolutionsverilogMlab2\arwz_pace.ise

X Fle Ed Vew Project Source Process Window telp

Design Summary]

RETERS

(E &

EEET TR EIET

YDE 0o vi®m 8 m

Ve

I FPGA Design Summary. [

Sources | Synbesi/plnentation

| | gDesion Overvien

Fawa_pace
=a chsEDDe 4g320

L var_clock (uart_clock v)

rocessor - kepsm3 (kepsm3.v)
rogram_rom - uclock (UCLOCK.Y)

euart - keuat_t keuart_ )
uf_0-bbffo_1643 (bbfifo_16+.v)
ceive - arl_rs(uat_.v)

euart - keuat_w euart )
uf_0-bbffo_1643 (bbfifo_16+.v)
) uart_clock.ucf (uat_clock.ucf)

2 Summany

2108 Progeries

[2) Timing Constaints

[ Pinout Repart

[2) Clock Repart
3o and Wanings

2 Syrihesis Messages
[ Tranclton Messages
[ Mep Messages
[@Place and Route Messages
(2 Timing Messages

[ Bitgen Messages
(2080 Cutent Messages
GDetied Repos

= 5o | gt | o |

Pracesses:

O AddExicting Source.
O Create New Saurce

T Create Schematic Symbol

(2] ViewHDL Souce

(2] ViewHDL Instanition Template

ARWZ_PACE Project Status

T
L B L ——
R L T —
----Wﬁf“‘*----WWWﬁﬁr‘*

Device Utiization Summary

Number of Sice Fiip Flops 147 9312

| Nober o i LUTs il sz |

_____

| b of S cortanin arl rlted g \ e \ e | [

8

8

R

[ Nber o Sices containg veled g | v

O Enable Message Fitering

00 Display Incremental Messsages
Enhanced Design Sunmery Corterts
O Show Enors

O Show Warings

00 Show Faiing Constaints

O Show Clack Repart

Gamnmiro -----------------
oo

B o | st \ \ \
2ot Tiing Repor | Numberused fr Dl Por bz I i \ I I
[ABitgen Report [ Numbet used for 324 éies T w2 I I
| Numberused s S riters [ & I I
| Number o bonced 08 [ 5| = | =
1108 i Flops [ 3] I I
| Number o lock bz I T il Bl
Projct Praeries | Humber o GCLKs [ 2 \ \ o |
b mstnimson]| e ‘ =

_____
|Addional /TAG gate courtforl0Bs | a0 [

Performance Summary

e
N L L  —
----@@@@f“‘*----F“““*

Detailed Reports

Sunthesis Repart Curert Tue May 16 122333 2006 385Wavmn s

| Tarloton gt~ | Curent | Tuehiay 161223452008 ‘n 8

Processes o
[FfPecon= | £ Dein umnay

Started : "Launching Design Suwanary”. |

.

Console | @Enors | j\Wamings | g FindinFies




[image: image28.png]2 |
He ES Vew It Fomst Iook Tobe Mindow e Type o cuestion for heb 51X
NEEHSSRITE 4 B 908 FREDBTT 0 @) el
stop3 +Loft (= Timesewonan ~ 10 «| B 7 U |[E] ERSAY % .|

[ 1 ' ' 1 ' 2 ' 3

" WPACT - C:/XUP MarkeisPLDs Workahops/coursesIve1_tpes
@ £e Edk view Ogerations Optons Quiput Debug Window Hebp

[PE XBOX #XZE 3 B0 ¥ R

<m0
N e fpga_flow - HyperTerminal DEX Bl
Fie Edt Yen Cal Iransfer e

- D@ » 3 DB &

with assigned configuration files

0 WwEBE o L EE§LEK

KCPSH3>_

ria communications cable to the Spartan-3 board and

1P ACT window, selest Program, and click <OK> inthe

> prompt in the hypestenminal windaw.

—————————

< 3
Connected 0:07:53 Autodetect 38400 SCROLL [ CAPS [ _| Copture

Figure 2-12. Serial Communication with PicoBlaze

-1

- Architecture Wizard & PACE e, xilinx. com/univ
Lab xup@siling.com

HEIT RORORS R S h: |
ipan- L [ Auoshapes~ \ N DO Ao BRSO L A== @df
Page 16 Sec 1 15/17 AL n33 Coli  REC TR S OV EnglshUs DK

€escemoc@ (W & |




[image: image29.png]4| J
DHe Edt Vew Dnset Fomat Dook T Wndow el Type 3 question for help v

NERSSRITE S B9 C 8 AE | Q91 100 - @ | dResd B
§ A4 step + Before + Times NewRoman « 10 - | B Z U [[E] |- - A -

[v]

© .
E T T
= FEX n with PicoBlaze
= (Fle Edt vew Cal Transfer Hel
il D& =3 DB &
= ]
3 KCPSM3> ti r
ine
= 00:06:53
| omiuniv 2-16
# KCPSHM3> [x.com
E
A
£l
& &
] |
jock Step 6
< 5 b teal-time clock, as specified in the
- | Connected 0:13:51 Auto detect 38400 SCROLL CAPSNOm | Cepture

O Tuter The command Thme & The command prompt to display the current time inthe farm of

Bhmmss

| - v pace iypertermiat
et ca i o
D&

T mm—————

Jeconaudosate msmamma sooms o oo w |
Figure 2-13. Display of Current Time

@ Enterthe command “slarny” at the command prompt o display the current alarm time in the
Form of Blumanss.

“wo it

=N RO R R o - |
Dowr s |awothepess N NOJOH A BRI L A== adf
Fage 17 Sec 1 16(17 At 25 ln10 Col 1 REC| TRK| [EXT | OVR [Englsh (U5 || O

esccmOcom M ] &) @ = [ @




[image: image30.png]PEle Edt Vew Imet Fomst ook Table Window Hep

NERHSSRITE S B9 G AR

| S @ oo - @) cread

§ A4 steps+ aria £ - Aria - 10

Type a question for help

2 B 3 B 4 B s

gri T EREE RN R RN
Operating the UART Real-Time Clock Step 6

A .

o
= - a
- I e ot s ot
=1 (Fle Edt vew Cal Transfer Hel
3 Sk ot o sy th o e it o o
- -~
5 KCPSH3) i
B ine ]
£ 06:06:53 EE)|
P B KCPSH3> alarn
3 06:00:00
Alarn OFF
A KCPSH3>
=)
E
= -
) e .
. | Connected 0:14:58 Auto detect 38400 SCROLL CAPSNOm | Cepture

@ Enterthe command “slarny” at the command prompt o display the cusrent alarm time in the
Form of blvmanss.

Note: the slarm s insctive

| < arwz_pace HyperTerminal
it ow co- Tt

De 53 0o &

0000000
KCPSHD> ine.
00:14:48

Figure 2-14. Display of alarm time and status

© Eutrth command "o o orth oo brcome e
-1 r

Architecture Wizard & PACE e, xilinx. com/univ 217
Lab xup@siling.com

A RO RO R R AR - |
Dow: s Jagoshpes- N NCIOS M AGEE - L-A-=S=cadf

“«o

Page 17 Sec 1 1617 AL 6" Ln 13 Col 1 REC TRK ENT OVR Engish(us X

EscerOcsE [ [iL2] fa o) = Il




_1080985683.doc
[image: image1.png]






_1148154534.doc
[image: image1.png]






